Модуль 4 Практика 3

Ученик (Подгруппа 1, Номер 18) – Иконников Родион Александрович

1)Используется схема schema-lab3 (проект с именем schema-lab3). Топология включает 5 коммутаторов (Layer2Switch-1 … Layer2Switch-5) и 6 ПК (PC1–PC6). Задача: создать две VLAN – VLAN 20 и VLAN 333. При передаче кадров VLAN 333 между коммутаторами использовать Native VLAN (т.е. кадры этой VLAN передаются без тегов 802.1Q).

Настройка Layer2Switch-1

enable

configure terminal

vlan 20

vlan 333

exit

interface range gi0/0-3 , gi1/0-3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 333

switchport trunk allowed vlan 20,333

no shutdown

exit

end

write

Настройка Layer2Switch-2

enable

configure terminal

vlan 20

vlan 333

exit

interface range gi0/0-1/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 333

switchport trunk allowed vlan 20,333

no shutdown

exit

end

write

Настройка Layer2Switch-3

enable

configure terminal

vlan 20

vlan 333

exit

interface gi1/0

switchport mode access

switchport access vlan 20

no shutdown

exit

interface gi1/1

switchport mode access

switchport access vlan 333

no shutdown

exit

interface range gi0/0-3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 333

switchport trunk allowed vlan 20,333

no shutdown

exit

end

write

Настройка Layer2Switch-4

enable

configure terminal

vlan 20

vlan 333

exit

interface gi1/0

switchport mode access

switchport access vlan 20

no shutdown

exit

interface gi1/1

switchport mode access

switchport access vlan 333

no shutdown

exit

interface range gi0/0-3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 333

switchport trunk allowed vlan 20,333

no shutdown

exit

end

write

Настройка Layer2Switch-5

enable

configure terminal

vlan 20

vlan 333

exit

interface gi1/0

switchport mode access

switchport access vlan 20

no shutdown

exit

interface gi1/1

switchport mode access

switchport access vlan 333

no shutdown

exit

interface range gi0/0-3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 333

switchport trunk allowed vlan 20,333

no shutdown

exit

end

write

2)ПК подключены к access-портам соответствующих VLAN.

| Устройство | VLAN | IP-адрес |
| --- | --- | --- |
| PC1 | 20 | 192.168.20.1/24 |
| PC2 | 333 | 192.168.33.2/24 |
| PC3 | 20 | 192.168.20.3/24 |
| PC4 | 333 | 192.168.33.4/24 |
| PC5 | 20 | 192.168.20.5/24 |
| PC6 | 333 | 192.168.33.6/24 |

Команды в VPCS:

PC1> ip 192.168.20.1/24

PC2> ip 192.168.33.2/24

PC3> ip 192.168.20.3/24

PC4> ip 192.168.33.4/24

PC5> ip 192.168.20.5/24

PC6> ip 192.168.33.6/24

С PC1 (192.168.20.1) выполняем ping на PC3 (192.168.20.3)

Результат: пакеты проходят, потерь нет.

С PC1 (VLAN 20) на PC2 (VLAN 333):

Вывод: трафик изолирован между VLAN на уровне L2, что соответствует требованиям.

3)Захват трафика произведён на транковом линке между Layer2Switch-1 и Layer2Switch-2.

При передаче ping между PC1 и PC3 (оба в VLAN 20) на транке появляются кадры с тегом 802.1Q, содержащим VID=20. При передаче ping между PC2 и PC4 (VLAN 333) на транке кадры идут без тега, так как Native VLAN настроен как 333. Коммутаторы отправляют такие кадры как обычные Ethernet-кадры.